Hellenic Accreditation System

# Annex G1/10 to the Certificate No.766-4 SCOPE of ACCREDITATION of the Hematology Department of Laboratories Division of General Hospital of Athens «G. GENNIMATAS»

| Tested materials / products | Types of tests/ Properties to be measured | Applied Methods / Techniques to be used |
| --- | --- | --- |
| Hematological tests |  |  |
| 1. Whole blood (EDTA) Whole blood (EDTA) (continued) Whole blood (EDTA) (continued) | Determination of 13 parameters | Automated Hematology Analyzers COULTER DxH 800* (1), (2), (3) |
|  | 1. Hemoglobin (HGB) | Photometric Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 2. Red Blood Cells (RBC) | Coulter principle (Electrical impedance) Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 3. Hematocrit (HCT%) | calculated as: RBC x MCV / 10 Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 4. Mean Corpuscular Volume (MCV) | Derived from RBC histogram Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 5. Mean Corpuscular Ηb (MCH) | calculated as: (Hb / RBC) x 10 Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 6. Mean Corpuscular Ηb Concentration (MCHC) | calculated as: (Hb / (RBC x MCV)) x 100 Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 7. White Blood Cells (WBC) | Coulter principle (Electrical impedance) Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 8. Neutrophils (absolute number and %) | VCS technology (Volume, Conductivity, Scatter) Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 9. Lymphocytes (absolute number and %) | VCS technology (Volume, Conductivity, Scatter) Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 10. Monocytes (absolute number and %) | VCS technology (Volume, Conductivity, Scatter) Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 11. Eosinophils (absolute number and %) | VCS technology (Volume, Conductivity, Scatter) Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 12. Basophils (absolute number and %) | VCS technology (Volume, Conductivity, Scatter) Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 13. Platelets (PLTs) | Derived from PLTs histogram Coulter principle (Electrical impedance) Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | Determination of 13 parameters | Automated Hematology Analyzer SYSMEX XN 1000 ROCHE* |
|  | 1. Hemoglobin (HGB) | SLS-hemoglobin method |
|  | 2. Red Blood Cells (RBC) | Hydrodynamically focussed DC detection method |
|  | 3. Hematocrit (HCT%) | RBC pulse height detection method (calculated) |
|  | 4. Mean Corpuscular Volume (MCV) | The MCV is calculated from the RBC and HCT, using the following equation: MCV (fl) = ((HCT(%) / RBC(x106μL)) x 10 |
|  | 5. Mean Corpuscular Ηb (MCH) | The MCH is calculated from the RBC and HGB, using the following equation: MCH (pg) = (HGB (g/dl) / RBC(x106μL)) x 10 |
|  | 6. Mean Corpuscular Ηb Concentration (MCHC) | The MCHC is calculated from the HCT and HGB, using the following equation: MCHC (g/dL) = HGB (g/dl) / HCT(%)) x 100 |
|  | 7. White Blood Cells (WBC) | Flow cytometry method using semiconductor laser |
|  | 8. Neutrophils (absolute number and %) | Flow cytometry method using semiconductor laser |
|  | 9. Lymphocytes (absolute number and %) | Flow cytometry method using semiconductor laser |
|  | 10. Monocytes (absolute number and %) | Flow cytometry method using semiconductor laser |
|  | 11. Eosinophils (absolute number and %) | Flow cytometry method using semiconductor laser |
|  | 12. Basophils (absolute number and %) | Flow cytometry method using semiconductor laser |
|  | 13. Platelets (PLTs) | Hydrodynamically focussed DC detection method |
|  | Determination of 13 parameters | Automated Hematology Analyzer CELLTAC G* |
|  | 1. Hemoglobin (HGB) | Photometric Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 2. Red Blood Cells (RBC) | Volume analysis Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 3. Hematocrit (HCT%) | calculated as: RBC x MCV /10 Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 4. Mean Corpuscular Volume (MCV) | Derived from RBC histogram Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 5. Mean Corpuscular Ηb (MCH) | calculated as: Hb / RBC Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 6. Mean Corpuscular Ηb Concentration (MCHC) | calculated as: Hb / RBC x MCV Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 7. White Blood Cells (WBC) | Volume analysis Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 8. Neutrophils (absolute number and %) | Flow cytometry Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 9. Lymphocytes (absolute number and %) | Flow cytometry Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 10. Monocytes (absolute number and %) | Flow cytometry Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 11. Eosinophils (absolute number and %) | Flow cytometry Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 12. Basophils (absolute number and %) | Flow cytometry Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 13. Platelets (PLTs) | Volume analysis Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
| 2. Blood smear morphology | Red Blood Cells’ Morphology / Platelets’ Count and Morphology/ White Blood Cell Differential Count (absolute number and %): Neutrophils, Lymphocytes, Monocytes, Eosinophils, Basophils, Abnormal Cells | Microscopic examination Ο.ΑΜ.620-3, version 4, 2/12/2019 |
| 3. Blood plasma (sodium citrate) | Determination of 4 parameters | Automated Coagulation Analyzers ACL TOP 750 CTS IL* (1), (2) |
|  | 1. Prothrombin time (PT) | Coagulometric Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 2. Activated Partial Thromboplastin time (APTT) | Coagulometric Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 3. Fibrinogen (FIB) | Modified Clauss method Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 4. DDimers (DD) | Immunoturbidimetric Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
| 4. Blood plasma (sodium citrate) | Determination of 5 parameters | Automated Coagulation Analyzer ACL TOP 750 CTS IL* (1) |
|  | 1. Antithrombin III (AT III) | Chromogenic Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 2. Protein C (Prot – C) | Chromogenic Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 3. Activated Protein C Resistance (APCR) | Coagulometric Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 4. Lupus Anticoagulant (LA) | Coagulometric Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
|  | 5. Free protein S antigen (Free Prot-S) | Immunoturbidimetric Ε.ΑΜ.640-2, ed. 1, 2/12/2019 |
| Molecular tests |  |  |
| 1. Whole blood (sodium citrate or EDTA) | Molecular thrombophilia testing for mutations FV Leiden G1691A, FII G20210A, MTHFR C677T, MTHFR A1298C | (i) DNA extraction, (ii) Multiplex amplification with PCR, (iii) reverse hybridization on membrane strips (DNA•STRIP technology CE-IVD kit Thrombotype plus v 3.0* της HAIN) Ο.ΑΜ.560-1, ed. 2, 2/12/2019 |

*Reference to the commercial name of a specific analyzer/kit,  refers to a specific analytical method and protocol

Site of assessment: Department laboratory facilities, General Hospital of Athens «G. Gennimatas»

154 Mesogeion Av. 11527, Athens, Greece.

Approved signatories: S. Chaniotaki, N. J. Tsagarakis, Ε. Goumakou, J. Chaliori, S. I. Papadimitriou.

Primary Sample Collection Site: General Hospital of Athens «G. Gennimatas» 154 Mesogeion Av. 11527, Athens, Greece.

This scope of Accreditation replaces the previous one dated 07.03.2024.

The Accreditation Certificate No. 766-4, to ELOT EN ISO 15189:2012, is valid until 07.09.2028.

- Athens, 28th May 2024

Christos Nestoras

CEO  of ESYD